PowerSDR 2.7.2 ke9ns Feature add list:

NCDXF Beacon Scanner: 
This feature gives you a direct method of determining radio wave propagation conditions(i.e. band conditions) for the 20m,17m, 15m,12m, and 10m bands. A System of 18 stations, around the world, transmitting (24hrs / day) in 10 second intervals on 5 frequencies (5 separate stations simultaneously) on 14.1mhz, 18.11mhz, 21.15mhz, 24.93mhz, and 28.2mhz. (Repeating every 3 minutes).Your PC clock must be accurately set to make sure PowerSDR matches up with the Beacon stations.
Under the “Spotter” window is a “Beacon Chk” button with Fast & Slow Scan options. You will see a list of Beacon stations (upper left corner) and the current 5 stations transmitting). With “Map Calls” checked, you will see all 18 stations (and each of their 5 frequencies). 

Slow Scan: Starts on 14.1mhz beacon and listens for 3minutes (1 complete loop) for 18 stations and records their signal strength, then moves to 18.11mhz, and so on, until 28.2mhz. 15 minutes total time. You can select the starting band 1 through 5. 1=14.1mhz up to 5=28.2mhz.
Fast Scan: Scans through all 5 Beacon frequencies (1 second per beacon) in a single 10 second interval, and records the signal strength on each frequency. It repeats this quick 5 frequency scan 18 times to get a complete Beacon map in 3 minutes, but is not as accurate as the Slow Scan.

Colors On the World Map: Gray = Not scanned yet. Violet = Currently Scanning, Red = Not detected, Yellow=Not detected/Weak, Orange=Light, Green=Strong.
On the SPOTTER window: All 18 stations x 5 frequencies are listed (total of 90 entries)
As the stations and frequencies are scanned, signal data is added to the SPOTTER entries.
-NA dBm = Not Scanned yet.  S9 -018dBm = S9 signal level, but only -18 dBm above the noise level.

At the Start of any Scan, the Mode is changed to CWU with filters set to 550hz - 650hz. This is to try and eliminate extraneous noise.  Prior operating Mode and Filter High/Low settings are restored when the scan is finished.

See http://www.ncdxf.org/beacon/index.html for further details

Automatic Antenna Control: Setup->CAT Control->Enable Rotor. You must select 1 side of a virtual COM port pair. The other side of the COM pair connects to DDUtil. DDUtil will automatically convert the beam heading into the proper format for your Rotor control.
Left Click on the BEAM HEADING of a DX spot in the Spotter window to Move Antenna
OR, Left Click on the Right Side of a DX Spot + CTRL Key in the Panadapter to Move Antenna 
In DDUtil open setup form->AntRtr and setup your antenna Rotor control(s).
 Then go to setup form->Ports->RCP1->Rotor Port and the “Other side” of the COM pair from PowerSDR.

MP3 output:  Scheduler/Recorder automatically saves only in MP3. Wave->QuickAudio Create MP3 checkbox, saves a WAV file and an MP3 file (For emailing the audio files to friends)

VFOA Slider: Left Click on PAN: text to free up the VFOA to slider around the display.

SWL BANDS: 14 standard Shortwave Listening bands, each with their own Bandstacking memories.
SWL SPOTTER: SWL.CSV database file (from eibispace.de) provides over 11000 shortwave broadcast, utility, and government frequencies all displayed directly on the Panadapter. CTRL+ Click on a station to open up a google search.
SWL Listing screen: Displays currently operating SWL stations by Frequency and is SEARCHABLE by Station Name. Left Click to go to frequency. Attempts to determine the mode (AM, DIGU, USB, CW). 
SWL Additional list: SWL2.csv provided by ke9ns adds SWL and HF Utility frequencies not found in the eibispace.de SWL.csv file.  PowerSDR stiches SWL.csv and SWL2.csv together when you run the SWL spotter.

DX Beam Heading: Enter in your Lat and Long next to your Call sign in the Spotter window, and see Beam heading from your Station. Map Checkbox to see beam headings on the Tracking Map.
DX CLUSTER: Using and internet and a DX Cluster URL address:port, and your callsign, DX station data shows up in a window based on time, checking for duplicates in frequency and call sign. Left click in this list to go directly to Frequency parsing out operating mode, and split. Right click to go to QRZ page  
DX SPOTTER: Displayed directly onto the Panadapter. CTRL + Click on spot in Pan, goes to QRZ page. SHIFT+Click on spot in Pan, alternates display from Spotted to  <Spotter>. 6m includes grid data
Grayline & Suntracker: World map plots Sun and/or Grayline, including Solstice and Space Weather data. Grayline has fill color and transparency settings and can be turned off separately.
Special Panafall mode: 80% map, 20% waterfall.
Mapping DX spots directly onto map which are live and selectable: CTRL+Click over red dot, to go directly to frequency, mode, and split. 
MEMORY list mapped directly to Panadapter. CTRL+Click on memory in Pan, to adjust filters, mode
MEMORY Added directly from Display: ALT+M key
MEMORY Hyperlinking: Drag/Drop a URL or file directly onto highlighted MEMORY (from memory screen). Right click on highlighted memory (from memory screen) to activate stored Hyperlink.
Memory (Scheduling & Recording):
You can now save a start Date & Time to change Frequency and optionally Record, repeating based on either the Week, or the Week of the Month. (i.e. Every Monday, or Last Monday of every month)

Bandstack Memory Window: Bandstack index# and size on main console window. Right click to open window. Add up to 9 memories (every band) with CTRL+Right Click on Band button.
Bandstack LOCK/UNLOCK feature for each memory in each Bandstack. Right Click on bandstack memory.

NOAA space weather on main console screen (SFI, A, K, SSN) in addition to PA Temp and Voltage.
Panadapter Fill color and Transparency.

New Analog Meters: Click on meters to cycle through all 3 (can also change colors). Also added RX “Signal peak” RX mode
2nd TX Meter Function: allows you to view 2 TX meter functions at the same time. Appears where the RX2 meter would be.

One Touch Automatic Small signal Panadapter Scale adjustment button: Left click on “Zoom:” text
One Touch Automatic Waterfall Adjustment button: (RX & TX) with offset adjustment sliders.
One Touch Automatic Panadapter Adjustment button: (RX & TX) with offset adjustment sliders.
Separate waterfall Low levels for TX display and RX2.
RX1 and RX2 both display in PanaFall mode at same time.
Waterfall History moves with Panadapter, so the history is always accurate. 
Gray waterfall (for viewing waterfall text and images)
TX WaterID: Transmit your call sign directly into the waterfall so anyone with a DSP can see your call sign (or transmit an bmp image)

RX1 mute allow VAC to continue, while still listening to RX2
CWX panel polls CW Key to interrupt message queue. Speed of CWX and CW key panel synced.
Direct Frequency entry in KHZ or MHZ (both VFOA and VFOB)
Native Griffin PowerMate USB knob support: no extra software
Native DJConsole USB support: no extra software

Quick Audio Save folder allows multiple saved recordings that can be played from the main console screen REC and PLAY buttons
PLAY button: Right click to see folder with your quickaudio files you can select from
Rec/Play ID feature: <REC>: Auto Record your voice. <PLAY>: Auto Play to Transmitter. For Contesting. 
Monitor function for AM/FM modes  (with limitations)
Monitor Pre and Post options (MONpr and MONps)
DSP Options (i.e. NR, ANF, NB)  reset to default buttons added.
TNF adjustable width
Band Scanner function
Continuum display mode for RX1
Corrected S9 reading above 30mhz
10khz wide TX Audio for 1500 and 3000
Opened up RX Audio to 15khz/side (30khz total)
Right Click on many buttons to go directly to setup screens (i.e. VAC1, NB, transmit profile)
Fix 6m lower limit for UK
Fix Full duplex (with RX2) operation not allowing VAC1 when transmitting on VFOB